|  | G:BOX mini 6 | G:BOX mini 9 | AZURE C-series |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | C400 | C500 | C600 |
| Applications |  |  |  |  |  |
| Gel Documentation | YES | YES | YES | YES | YES |
| Fluorescence | YES | YES | YES | YES | YES |
| Colorimetry/ densitometry | YES | YES | YES | YES | YES |
| Chemi- luminescence | YES | YES | YES | YES | YES |
| Western fluorescence | Optional | Optional | YES | NO | YES |
| IR | Optional | Optional | NO | YES | YES |
| Camera |  |  |  |  |  |
| Image resolution | CCD, 6MP, 2750x2200 18MP effective 73% QE at 425nm Peak QE 77% | CCD, 9MP, 3380x2704 27MP effective 73% QE at 425nm Peak QE 77% | CCD 8.3 MP, 5.4um x 5.4um pixels, 3348 x 2574, peak QE 60% (40% at 400nm) |  |  |
| Bit depth (gray levels) | 16 bit, 65,536 |  | 16 bit, 65,536 |  |  |
| Dynamic range | 4.8 OD |  | 4.8OD |  |  |
| Max exposure time | 23hrs 59mins |  | Unknown |  |  |
| Lens |  |  |  |  |  |
| Type | Motor driven, f/0.95 |  | Motorised f0.95 (25mm) |  |  |
| Max sample size | 15 x 12cm |  | 20 x 15cm |  |  |
| Min sample size | 10 x 8cm |  |  |  |  |
| Sample Platform |  |  |  |  |  |
| Type | Motor driven multi position sample platform |  | Motor driven zoom lens |  |  |
| Filter Selector |  |  |  |  |  |
| Filter positions | 7 positions - Motor driven |  | 7 positions - Motor driven |  |  |
| Lighting Options |  |  |  |  |  |
| Epi-white light | Yes |  | Yes |  |  |
| HI-LED (red, blue, green) | Optional |  | Optional |  |  |
| HI-LED (red, infrared) | Optional |  | Optional |  |  |
| HI-LED (red, blue, green, infrared) | Optional |  | Optional |  |  |
| Blue converter screen | Optional |  | Optional |  |  |
| Visible light converter | Optional |  | Optional |  |  |
| Transilluminator | Optional for 302nm, 365nm, 254nm |  | Yes |  |  |
| Hardware |  |  |  |  |  |
| Dimensions (WxHxD) | 40 x 64 x 52 cm |  | 38 x 45 x 30 cm |  |  |
| Operating System | Option for external PC or mounted touch screen PC |  | Inbuilt PC |  |  |
| Software |  |  |  |  |  |
| Image acquisition | Yes |  | Yes |  |  |
| Analysis | Yes – unlimited free copies of GeneTools |  | Yes |  |  |
| Warranty |  |  |  |  |  |
|  | 3 years |  | 1 Year |  |  |